Gully erosion

Gully erosion is the removal of soil along drainage lines by surface water runoff. Unless steps are taken to stabilise the disturbance, gullies will continue to move by headward erosion or by slumping of the side walls.

It's far easier and more economical to do repair work in the early stages of newly formed gullies. Large gullies that have been left unchecked are difficult and costly to repair.

# The cause of gully erosion

Gully erosion occurs when water is channelled across unprotected land and washes away the soil along the drainage lines. Under natural conditions, runoff is moderated by vegetation, which generally holds the soil together, protecting it from excessive runoff and direct rainfall.

Excessive clearing, inappropriate land use and compaction of the soil caused by grazing often means the soil is left exposed and unable to absorb excess water. Surface runoff then increases and concentrates in drainage lines, allowing gully erosion to develop in susceptible areas.

Some of the problems caused by gully erosion include:

dissection of the property causing access and management difficulties

loss of productive land (gullies often occur in the most productive area of the catchment)

reduced amenity and property values, including destruction of farm improvements, such as fences or tracks

discolouration of water supply and sedimentation of waterways, dams and lower paddocks

provides a harbour for vermin.

# Preventing gully erosion

In most cases gullies can be prevented by good land management practices aimed at maintaining even infiltration rates and a good plant cover.

Strategies for preventing gully erosion include:

maintaining remnant vegetation along drainage lines and eliminating grazing from these areas

increasing water usage by planting deep-rooted perennial pastures, trees, or an appropriate mixture of both thus maintaining healthy, vigorous levels of vegetation

identifying drainage lines as a separate land class in which vegetation needs to be protected

immediate stabilisation of sheet or rill erosion

vermin control

ensuring runoff from tracks is evenly distributed across paddocks ton dissipate its energy

maintaining high levels of organic matter in the soil

avoiding excessive cultivation (fence and manage the land according to its capability)

# Control measures for gully erosion

To be effective, gully control needs to be tackled in two ways:

fixing the problems in the catchment

stabilising the gully itself.

## Catchment works

The objective of catchment works is to reduce and divert the flow of water into stable drainage lines. This can be achieved by:

increasing infiltration rates and water uptake by plants

by diverting and storing water.

A practical way to begin is to subdivide the catchment into appropriate land classes and then apply grazing and cropping practices most suited to each class. The development of a land management or whole farm plan is an ideal way of identifying these issues.

Strategies for stabilising the catchment include:

cooperatively tackling the problem by the formation of a landcare group — this could be be the most effective method where the source of the problem is spread over several properties

diversion of water away from erosion prone gullies (for example, with diversion banks) — this disperses the erosive power of the water over well-vegetated areas

contour cultivation where possible to slow down runoff and spread the water over a wide area

maintaining farm tracks and culverts so that drainage is evenly dissipated and prevented from concentrating along any section

using trees and deep-rooted perennial pastures to assist in both utilising excess water and reducing runoff.

## Stabilising gullies

The objective is to divert and modify the flow of water moving into and through the gully so that scouring is reduced, sediment accumulates and revegetation can proceed. Stabilising the gully head is important to prevent damaging water flow and headward erosion.

A variety of options can be used to get the water safely from the natural level to the gully floor, such as:

grass chutes

pipe structures

rock chutes

drop structures.

Structures might also be required along gully floors since some grades can be quite steep and allow water to rush down under peak flows, ripping away soil and vegetation. These may take the form of rock barrages, wire netting or logs across gullies.

Sediments held in the water will then be deposited along the flatter grades as a result of slower water flow, allowing vegetation to re-establish.

If erosion control and revegetation work is undertaken, then damaged areas should be fenced off from stock, until restoration is complete.

Dams can also be constructed to slow the flow of water into the gully head, but special care needs to be taken to get the overflow water back into the gully floor safely.

# Don't fill gullies with solid objects

Don't fill eroded gullies with solid objects such as old drums, car bodies or concrete. This only creates further erosion by directing water around such objects and removing more soil.

Accessibility

If you would like to receive this publication in an accessible format, please telephone the Customer Service Centre on 136 186, TTY 1800 122 969, or email customer.service@ecodev.vic.gov.au.